CG2271 Real Time Operating Systems

Lab 1 – ARM Cortex M0+ GPIO Programming

- Introduction

In this lab we will use what we have learnt about the Arm Cortex M0+ GPIO subsystem to drive the LED that is on the Freedom MCXC444 board.

- Submission Instructions

Do this lab during the slot allocated to you, and submit the report by Friday 5 September 2025, 11.59 pm to your respective bin on Canvas.

You can do the demo at the end of the lab session or the start of the following lab session.

- Understanding the FRDM-MCXC444 Board

Looking at the Schematics of the board in the SPF-93440_A.pdf file, we see how the RGB LED is connected to the MCX444:

We see the following:

| Color | Pin |
| --- | --- |
| Red | PTE31 |
| Green | PTD5 |
| Blue | PTE29 |

Thus, we must enable ports D and E in the System Integration Module (SIM). Additionally we look at how the LEDs are connected:

Question 1 (1 mark)

Are the LEDs active high or active low (i.e. should the GPIO pin be driven high or low to turn on the LED)? Explain your answer. (2 marks)

The FRDM-MCXC444 board has two switches, the WAKEUP button at the top and the NMI button at the bottom. We will use this switch:

The circuit diagram shows that this switch (SW3) is connected to PTA4:

Question 2 (1 mark)

Is SW3 active high or active low? Explain your answer. (2 marks)

- Explore Programming the GPIO Ports

A project called “Blinky” has been provided to you in the Blinky directory which was included in CG2271Lab1.zip. Let us explore this project a bit:

- Select “Import project(s) from file system” from the Quickstart Panel at the bottom left of the MCUXpresso screen:

Click Browse next to “Project directory (unpacked)”:

Navigate to the directory where you unzipped the Blinky project. Click Open, then Finish.

If you get a box like:

Click “Yes To All”.

The Blinky project will now be in the Project Explorer. Click Source then Blinky.c to open the source file.

- Examine the following code:

Explain what each of the following lines do (1 mark each – total 4 marks)

Question 3 (4 marks)

| Line | Brief Explanation |
| --- | --- |
| SIM->SCGC5 ¦= SIM_SCGC5_PORTE_MASK; |  |
| PORTE->PCR[RED_PIN]&= ~PORT_PCR_MUX_MASK; |  |
| PORTE->PCR[RED_PIN]= PORT_PCR_MUX(1); |  |
| GPIOE->PDDR ¦= (1 << RED_PIN); |  |

- Create a New Project

Create a new C/C++ project and call it CG2271Lab2. See Lab 1 if you are not sure how to create a new project in MCUXpresso.

- Programming the GPIO Ports

In this part of the lab we will program the FRDM-MCXC444 board so that We will now program the GPIO ports for the LEDs and the SW3 switch.

- Delete the contents of the CG2271Lab2.c source file, and copy over the contents from the lab2.c source file provided in this ZIP file.

- Fill in the body for initGPIO to initialize the GPIO for the 3 LEDs and SW3. (4 marks):

Question 4 (4 marks)

void initGPIO() {

}

- Fill in the code in ledOn to turn on the respective LEDs.  Cut and paste your code into the space below:

Question 5 (3 marks)

void ledOn(TLED led) {

switch(led) {

case RED:

// Code to turn on RED LED

break;

case GREEN:

// Code to turn on GREEN LED

break;

case BLUE:

// Code to turn on BLUE LED

break;

}

}

- Fill in the code in ledOff to switch off the LED indicated. Cut and paste your code here (3 marks)

Question 6 (3 marks)

void ledOff(TLED led) {

switch(led) {

case RED:

// Turn off RED led here

break;

case GREEN:

// Turn off GREEN led here

break;

case BLUE:

// Turn off BLUE led here

break;

}

}

- We will now amend main to read the switch. Locate these lines:

int count = 0;

while(1) {

// Switch is active low

if(// Code to read switch) {

switch(count) {

Change “// Code to read switch” to read SW3.  This line should evaluate to TRUE if SW3 is pressed. Cut and paste your code below (1 mark)

Question 7 (1 mark)

while(1) {

// Switch is active low

if(        ) {

switch(count) {

case 0:

ledOn(RED);

break;

case 1:

ledOn(GREEN);

break;

…

- Demo (3 marks)

Demo your code to your TA either by the end of the Lab 2 session, or at the start of Lab 3.